PART 2 - PRODUCTS

- 2.1	TURFGRASS SOD:

        - Description: Desert GreenTM Bluegrass Sod suitable for low maintenance areas, reclamation, roadsides, pastures, and large low-traffic parks of uniform density, color, and texture, strongly rooted, and capable of vigorous growth and development when planted.

        - Desert GreenTM Bluegrass Sod specifications include but are not limited to:

- Shall be grown from a 5 seed blend utilizing cultivars that exceed normally accepted industry criterion.

- Shall contain no rye grass cultivars.

- Shall have a maximum 50# per acre seeding rate.

- Shall have a minimum multi-year root mass with strong rhizome development.

- Shall be certified noxious weed free.

- Shall be Water Wise Turf:

  - Shall require no more than 30” annual supplemental water during establishment and no more than 20” annual supplemental water after establishment.

  - Shall contain a strong root base capable of root penetration to a minimum depth of 8” into landscape soil.

- Growing medium shall be a sandy loam soil with low clay content.

- Shall be grown without netting.

- Shipping mow height shall be 2-3”.

- Shall be supplied by Chanshare Farms – 10785 W. 12800 N., Tremonton, UT 84337.