PART 2 - PRODUCTS

- 2.1	TURFGRASS SOD:

        - General Information

          - Provide bluegrass sod that is certified, approved, Number 1 Quality/Premium, including limitations on thatch, weeds, diseases, nematodes, and insects, complying with TPI's "Specifications for Turfgrass Sod Materials" in its "Guideline Specifications to Turfgrass Sodding."

          - Furnish bluegrass sod suitable for low maintenance areas, reclamation, roadsides, pastures, and large low-traffic parks of uniform density, color, and texture, strongly rooted, and capable of vigorous growth and development when planted.

        - Bluegrass Sod

- Sod specifications include but are not limited to:

  - Shall be grown from a 5 seed blend of bluegrass utilizing cultivars that exceed normally accepted industry criterion.

  - Shall contain no rye grass cultivars.

  - Shall have a maximum 50# per acre seeding rate.

  - Shall be certified noxious weed free.

  - Shall have a multi-year root mass with strong rhizome development.

  - Shall be Water Wise Turf:

    - Shall require no more than 30” annual supplemental water during establishment and no more than 20” annual supplemental water after establishment.

    - Shall contain a strong root base capable of root penetration to a minimum depth of 8” into landscape soil.

  - Growing medium shall be a sandy loam soil with low clay content.

  - Shall be grown without netting.

  - Shipping mow height shall be 2-3”.