## TURFGRASS SOD:

    - General Information

      - Provide natural living turfgrass sod that is certified, approved, Number 1 Quality/Premium, including limitations on thatch, weeds, diseases, nematodes, and insects, complying with TPI's "Specifications for Turfgrass Sod Materials" in its "Guideline Specifications to Turfgrass Sodding."

      - Furnish natural living TWCA Qualified Bluegrass sod suitable for low maintenance areas, reclamation, roadsides, pastures, sports fields, and large parks of uniform density, color, and texture, strongly rooted, and capable of vigorous growth and development when planted.

    - Sod specifications include but are not limited to:

      - Shall be grown from a TWCA Qualified 5 seed bluegrass blend utilizing cultivars that exceed normally accepted industry criterion.

      - Shall contain no rye grass cultivars.

      - Shall have a maximum 50# per acre seeding rate.

      - Shall have a minimum multi-year root mass with strong rhizome development.

      - Shall be certified noxious weed free.

      - Shall be TWCA Qualified drought tolerant turfgrass.

        - Shall require no more than 30” annual supplemental water during establishment and no more than 20” annual supplemental water after establishment.

        - Shall contain a strong root base capable of root penetration to a minimum depth of 8” into landscape soil.

      - Growing medium shall be a sandy loam soil with low clay content.

      - Shall be grown without netting.

      - Shipping mow height shall be 2”.