PART 2 - PRODUCTS

## TURFGRASS SOD:

    - General Information

      - Provide natural living turfgrass sod that is certified by the UCIA, approved by the Utah Nursery Inspection Program, Number 1 Quality/Premium, including limitations on thatch, weeds, diseases, nematodes, and insects, and complying with TPI's "Specifications for Turfgrass Sod Materials" in its "Guideline Specifications to Turfgrass Sodding."

      - Furnish natural living turfgrass sod suitable for low maintenance areas, reclamation, roadsides, pastures, golf courses, cemeteries, sports fields and large parks of uniform density, color, and texture, strongly rooted, and capable of vigorous growth and development when planted.

    - Tahoma 31 Bermudagrass Sod specifications include but are not limited to:

      - Shall be grown from foundation material secured from a registered Tahoma 31 grower.

      - Shall come from foundation material having 200-300 bushel per acre sprig installation rate.

      - Shall have a minimum multi-year root mass with strong stolon development

      - Shall be certified noxious weed free.

      - Shall be Triploid hybrid Bermudagrass approved by the UCIA and .

        - Requiring no more than 20” annual supplemental water during establishment and no more than 15” annual supplemental water after establishment.

        - Containing a strong root base capable of root penetration to a minimum depth of 18” into landscape soil.

      - Growing medium shall be a sandy loam soil with low clay content.

      - Shall be grown without netting.

      - Shipping mow height shall be 2”.

      - Shall be supplied by Chanshare Farms – 10785 W. 12800 N., Tremonton, UT 84337